# Name & Student ID

Marcus Chow - 20576816

# Title

Drawing Robot Software – System Manual

# Software Description

- This program reads user input text and processes the text as G-Code, which will be transmitted to a writing robot.

- The robot is controlled by a RS232 microcontroller, which this program will be communicating with.

- This program is broken down in 4 sections:

  - Read single stroke font data and store it in memory

  - Read the text in the file and save as an array

  - Extract stroke data in order of the written text character

  - Convert the arranged stroke data into G-Code

- The program prompts for font size before writing as G-Code, so the writing text output can be modified. Below are 3 output G-Code instances with initial height of 4mm, 7mm and 10mm writing “The quick brown fox jump over the lazy dog”, running through a G-Code simulation program. [G-Code Simulator made by nraynaud in Github, https://nraynaud.github.io/webgcode/]

- This program also automatically line break when the word exceeds the writing area (default as 100mm, modify if needed), to make sure the entire word is written without any breaks.

# Project Files

Files:

- Main.c – The main program that sends the G-Code data

- Main_fn.c – The .c file that includes all the functions

- Main_fn.h – The header for the main_fn.c file

- Rs232.c – the program file that includes the functions to process communication to the RS232

- Rs232.h – header file of rs232.c

- Serial.c – program that configure the communication between the program and the RS232 microcontroller, which controls the drawing output.

- Serial.h – header file of serial.c

- SingleStrokeFont.txt – The file that stores the drawing data per ASCII code

- Test.txt – Input text sequence to be drawn by the robot

- SystemManual_MNYC.docx – The manual file you are currently reading

- RobotFlowchartMNYC_Updated.pdf – the flowchart that shows the program flow

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| SingleStrokeFont.txt | Text file | The file that stores the drawing data per ASCII code |
| Test.txt | Text file | Input text sequence to be drawn by the robot |
| SSFdata | Int** | This forwards the SSF data as array obtained from SingleStrokeFont.txt |
| rows | int | Number of rows from SingleStrokeFont.txt |
| buffer_seq | Char* | The character sequence obtained by the text typed in test.txt |
| length | int | Length of the character sequence |
| Init_height | int | User input character height |
| Stroke_data | int | The arranged array returned by putting data from SSFdata 2D array in the buffer_seq character sequence |
| strokes | int | Total number of strokes to be processed to the robot, also the total number of rows of stroke_data |
| drawG | int | Integer variable to distinguish when to pen down |
| X_pos | Float | The x position extracted from stroke_data array to be output to G-Code sequence |
| Y_pos | float | The y position extracted from stroke_data array to be output to G-Code sequence |

# Functions

Here are the functions that are used to generate the G-Code sequence:

int** read_SSF(const char* filename, int* rows)

Parameters:

Filename – Input pointer of the file to be opened

Rows – Output pointer to return number of rows in the file

Return value – Returns the pointer SSF data array, 0 if file is unavailable or restricted

char* file_to_buffer(const char* filename, int* length)

Parameters:

Filename – Input pointer of the file to be opened

length – Output pointer to return number of characters in the file

Return value – Return pointer character buffer array, 0 if file is unavailable or restricted

int extract_and_print_strokes

(

const char* buffer,

int number_of_chars,

int** SSF,

int K,

int stroke_data[][3],

int init_height

)

Parameters:

buffer – Input pointer for the ASCII buffer array processed from the file_to_buffer function

number_of_chars – Input pointer to return number of characters in the file

SSF – Input pointer for input the SSF data array processed from the read_SSF function

rows_process – Input number of rows to process

Stroke_data[][3] – Output stroke data array processed in this function

Init_height – Input initial height in mm

Return value – returns stroke_index, number of strokes to be processed to robot

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| Read_SSF() | Count the number of lines in text file | SingleStrokeData.txt, int ch, int k | k will be one less than the number of lines in .txt, as the last line doesn’t have line break |
| Read_SSF() | Write data from text file into 2D array | SingleStrokeData.txt, int** SSF[][], int* rows | Data in the array SSF should be identical to the text file |
| File_to_buffer() | Count the number of characters in text file | Test.txt, int c, int total_chars | Total_chars will be the sum number of letters, spaces, enter and null written in the text file. |
| File_to_buffer() | Write data from text file into array of char | Test.txt, char* buffer_f[], int c | Data in buffer_f array should be the ASCII sequence of the text in test.txt |
| Extract_and_print_strokes() | New line char shifts | Int enter, int totat_chars_out, float current_width_mm, const char* buffer | Line break character should shift Y downwards and reset X offset |
| Extract_and_print_strokes() | Space character | Int total_chars_out, float current_width_mm, const char* buffer | Space character should offset x value by one character |
| Extract_and_print_strokes() | Measure word width | int number_of_chars, float word_width_mm, const char* buffer, int init_height | Word_width_mm should show the width of the current word by 18 * initial height per character |
| Extract_and_print_strokes() | Check if the word fits | float current_width_mm, float word_width_mm, const float max_width_mm, int enter, int totat_chars_out | Line break and reset x offset if total width exceeds max width |
| Extract_and_print_strokes() | Search index row | const int sep_value = 999 int** SSF[][] const char* buffer int rows_process | For 999 C N, search for 999 and character within buffer = C, output N, number of rows to be read |
| Extract_and_print_strokes() | Write N rows of SSF | int stroke_data[][3] int** SSF[][] int totat_chars_out int enter int stroke_index | Stroke_data array should have the corresponding SSF data assigned to the character. |
| main() | Pen down pen up G-Code association | int drawG int stroke_data[][] | Third column of stroke data array should assign with S1000 and G1 when = 1 and S0, G0 when = 0 |
| main() | X and Y G-Code association | Float x_pos float y_pos int init_height int stroke_data[][] | First and second column of stroke data should be scaled by the initial height and first column as X, second column as Y |
| main() | Output G-Code coordinates | Float x_pos float y_pos int drawG | If both 2 tests above succeed, the output column should comply to the G X Y G-Code format. |
| main() | Output G-Code to “RS232” | All variables above | The G-Code output should output the text in test.txt in the G-Code simulator or the writing robot. |

# Flowchart(s)

- The flowchart is included within this repository as a separate PDF document